Scope of Work – Project 5

Background Information:

Project Title: Automated Pavement Balance Mix Design

Team Members: Christopher Moore

Initial submission for review:

Submitted to: Nebraska Department of Transportation

Project Goals:

The goal of this project is to provide a detailed analysis of a proposed asphalt mixture. This analysis will provide 4 key features: an aggregate graduation plot, volumetric properties calculator, IDEAL – CT, and rutting analysis.

Together, all these modules will provide the user(s) with a deeper understanding of whether their proposed mix design meets the required standards set by the Nebraska Department of Transportation.

Project Tasks:

Task 1: Aggregate Gradation

This task will consist of a code that will read inputted sieve data for each aggregate and their corresponding blend percentages. It will take the inputted day and provide the user with a gradation curve, which will be compared to acceptable NDOT specifications.

Task 2: Volumetric Properties Calculator

Task 2 will utilize equations found within the provided excel file to calculate specified properties. If the calculated properties does not match the allowable 6.5 to 7.5% air voids, it will report “invalid performance testing” to the user.

Task 3: IDEAL – CT

Task 3 will introduce the ability to take raw test data and generate a Load vs. Displacement comparison. The code will calculate performance values such as peak load, displacement at 75%, post-peak slope, and area underneath the established curve. All this information determined by the code will result in computing the CTindex and displaying a Load vs. Displacement plot.

Task 4: Rutting Analyzer

Task 4 will analyze Hamburg Wheel Tracking Test data by reading wheel cycles and the rut depth measurements. The proposed code will also identify the stripping inflection point, which will indicate when rutting begins to increase rapidy due to moisture damage. It will also determine the cycle count at which the rut depth reached 12.5 mm. Additionally, the code will determine whether the mix design survives 20,000 cycles and will provide a category of pass or fail and create a plot comparing Rut Depth vs. Cycles.

Task 5: User Guide

A guide will be provided, which will have information on how the user should input values. Additionally, background information will be provided so all users, regardless of background, will be able to understand and utilize the proposed code.